### 1. Project Overview

- Objective: Design a robust, scalable, and secure network for a new building with 600 users across 3 floors.

- Key Components:

  - Floors:

    - First Floor: Sales & Marketing (120 users), Human Resource & Logistics (120 users).

    - Second Floor: Finance & Accounts (120 users), Administration & Public Relations (120 users).

    - Third Floor: ICT (120 users), Server Room (12 devices).

  - Network Requirements:

    - Hierarchical design with redundancy.

    - Wireless networking for each department.

    - VLAN segmentation and subnetting.

    - Dynamic IP addressing via DHCP (except server room).

    - Inter-VLAN routing using multilayer switches.

    - OSPF as the routing protocol.

    - SSH for remote access.

    - Port security for Finance & Accounts.

    - PAT with ACLs for internet access.

    - Connectivity to two ISPs for redundancy.

### 2. Hierarchical Network Design

- Core Layer:

  - Routers: R1 and R2 for redundancy and ISP connectivity.

- Distribution Layer:

  - Multilayer Switches: MLS-W1 and MLS-W2 for inter-VLAN routing and redundancy.

- Access Layer:

  - Access Switches: Connect end-user devices.

### 3. Redundancy

- Router Redundancy: Two routers (R1 and R2) ensure high availability.

- Switch Redundancy: Two multilayer switches (MLS-W1 and MLS-W2) provide redundancy.

- ISP Redundancy: Two ISPs ensure internet connectivity even if one fails.

### 4. VLAN Segmentation

- Each department is assigned a unique VLAN:

  - VLAN 10: Sales & Marketing.

  - VLAN 20: Human Resource & Logistics.

  - VLAN 30: Finance & Accounts.

  - VLAN 40: Administration & Public Relations.

  - VLAN 50: ICT.

  - VLAN 60: Server Room.

### 5. Subnetting

- Base network: 172.16.1.0.

- Subnets:

  - First Floor:

    - Sales & Marketing: 172.16.1.0/25.

    - Human Resource & Logistics: 172.16.1.128/25.

  - Second Floor:

    - Finance & Accounts: 172.16.2.0/25.

    - Administration & Public Relations: 172.16.2.128/25.

  - Third Floor:

    - ICT: 172.16.3.0/25.

    - Server Room: 172.16.3.128/28.

### 6. Dynamic IP Addressing

- DHCP Servers: Located in the server room.

  - Assign IP addresses dynamically to all devices except server room devices (static IPs).

### 7. Inter-VLAN Routing

- Multilayer Switches: MLS-W1 and MLS-W2 handle inter-VLAN routing.

  - Allow devices in different VLANs to communicate.

### 8. Routing Protocol

- OSPF (Open Shortest Path First):

  - Used for routing between routers and switches.

  - Ensures efficient route advertisement and redundancy.

### 9. SSH for Remote Access

- SSH (Secure Shell):

  - Configured on all routers and multilayer switches.

  - Provides secure remote access for network management.

### 10. Port Security

- Finance & Accounts Department:

  - Only one device per switch port.

  - Sticky Method: Learns and stores MAC addresses.

  - Violation Mode: Shuts down the port if unauthorized devices connect.

### 11. PAT with ACLs

- PAT (Port Address Translation):

  - Uses the outbound router interface IPv4 address.

  - ACLs (Access Control Lists):

    - Control internet access by defining rules for traffic.

### 12. Wireless Networking

- Each department has its own wireless network.

- Wireless Access Points:

  - Provide wireless connectivity to users.

### 13. Technologies Used

- Cisco Packet Tracer: For network design and simulation.

- Routers: Cisco routers (R1 and R2).

- Multilayer Switches: Cisco MLS-W1 and MLS-W2.

- Access Switches: For connecting end-user devices.

- Wireless Access Points: For wireless networking.

- Protocols:

  - OSPF: Routing.

  - SSH: Secure remote access.

  - PAT: Internet access translation.

  - DHCP: Dynamic IP allocation.

  - VLANs: Network segmentation.

### 14. Key Configurations

- Basic Device Settings:

  - Hostnames, console password, enable password, banner messages, disable IP domain lookup.

- VLAN Assignment:

  - VLANs assigned to each department.

- Switchport Security:

  - Configured for Finance & Accounts department.

- OSPF Configuration:

  - On routers and multilayer switches.

- Static IP for Server Room:

  - Devices in the server room have static IPs.

- DHCP Server Configuration:

  - DHCP pools for each department.

- Inter-VLAN Routing:

  - Configured on multilayer switches.

- PAT + ACL:

  - Configured on routers for internet access.

### 15. Testing and Verification

- Ping Tests:

  - Ensure devices can communicate across VLANs and with the internet.

- SSH Access:

  - Verify secure remote access to routers and switches.

- Port Security:

  - Test by connecting unauthorized devices to Finance & Accounts ports.

- PAT and ACL:

  - Verify internet access and traffic control.

### 16. Conclusion

- The network design ensures high availability, security, and scalability.

- It meets current business needs and is future-proofed for growth.

- The use of redundancy, VLANs, OSPF, and SSH ensures a robust and secure network.

### Key Points to Remember

- Hierarchical Design: Core, Distribution, and Access layers.

- Redundancy: Routers, switches, and ISPs.

- VLANs: Each department has its own VLAN.

- Subnetting: Efficient IP allocation using 172.16.1.0.

- DHCP: Dynamic IP assignment for all devices except server room.

- Inter-VLAN Routing: Handled by multilayer switches.

- OSPF: Routing protocol for efficient route advertisement.

- SSH: Secure remote access to devices.

- Port Security: Finance & Accounts department.

- PAT + ACL: Internet access and traffic control.

- Wireless Networking: Each department has its own wireless network.

### Common Viva Questions

- What is the purpose of VLANs in this network?

  - VLANs segment the network by department, improving security and reducing broadcast traffic.

- Why is OSPF used instead of other routing protocols?

  - OSPF is efficient, scalable, and supports fast convergence, making it ideal for large networks.

- How does PAT work in this network?

  - PAT allows multiple devices to share a single public IP address for internet access, conserving IP addresses.

- What is the role of multilayer switches?

  - They handle both switching and routing, enabling inter-VLAN communication.

- Why is port security important for the Finance & Accounts department?

  - It ensures only authorized devices can connect, enhancing security.

- How does DHCP simplify IP address management?

  - DHCP automatically assigns IP addresses to devices, reducing manual configuration errors.

- What is the significance of having two ISPs?

  - It provides redundancy, ensuring internet connectivity even if one ISP fails.

- How do you test the network after configuration?

  - Use ping tests, SSH access, and verify port security and PAT functionality.

### Final Tips

- Practice Configurations: Be familiar with basic commands for VLANs, OSPF, SSH, and PAT.

- Understand Subnetting: Be able to explain how subnetting works and why it’s used.

- Explain Redundancy: Be clear on how redundancy improves network reliability.

- What is the purpose of using multilayer switches in this network?

  - Multilayer switches handle both switching (Layer 2) and routing (Layer 3), enabling inter-VLAN communication and reducing the need for separate routers.

- Why is OSPF preferred over static routing in this network?

  - OSPF dynamically adjusts to network changes, provides faster convergence, and reduces manual configuration errors compared to static routing.

- What is the role of the DHCP server in this network?

  - The DHCP server automatically assigns IP addresses to devices, ensuring efficient IP management and reducing manual configuration.

- How does VLAN segmentation improve network performance?

  - VLANs reduce broadcast traffic by isolating traffic within departments, improving network efficiency and security.

- What is the purpose of the "sticky" method in port security?

  - The sticky method dynamically learns and stores the MAC address of the first device connected to a port, ensuring only that device can connect in the future.

- How does PAT help in conserving IP addresses?

  - PAT allows multiple devices to share a single public IP address for internet access, reducing the need for multiple public IPs.

- What is the significance of having two ISPs in this network?

  - Two ISPs provide redundancy, ensuring internet connectivity remains available even if one ISP fails.

- How do you ensure secure remote access to network devices?

  - SSH (Secure Shell) is configured on routers and switches to provide encrypted remote access, preventing unauthorized access.

- What is the difference between a router and a switch?

  - Router: Operates at Layer 3 (Network Layer), routes data between different networks, and uses IP addresses.

  - Switch: Operates at Layer 2 (Data Link Layer), forwards data within the same network, and uses MAC addresses.

- What is the purpose of the server room VLAN?

  - The server room VLAN isolates critical server devices, providing enhanced security and management for server resources.

- How do you verify that OSPF is working correctly?

  - Use commands like show ip ospf neighbor to check OSPF neighbor relationships and show ip route to verify OSPF routes.

- What is the purpose of the "ip helper-address" command?

  - The ip helper-address command forwards DHCP requests from clients to the DHCP server, enabling dynamic IP assignment across VLANs.

- How do you handle a port security violation?

  - The port is shut down if an unauthorized device connects, preventing unauthorized access.

- What is the difference between a VLAN and a subnet?

  - VLAN: A logical grouping of devices within a network, regardless of physical location.

  - Subnet: A division of an IP network, defined by a range of IP addresses.

- Why is inter-VLAN routing necessary?

  - Inter-VLAN routing allows devices in different VLANs to communicate with each other, which is essential for departments to share resources.

- What is the purpose of the "banner motd" command?

  - The banner motd command displays a message when someone logs into the device, often used for security warnings.

- How do you ensure that the network is future-proofed?

  - By using scalable technologies like VLANs, OSPF, and multilayer switches, the network can accommodate future growth and changes.

- What is the difference between a public IP and a private IP?

  - Public IP: Used for devices accessible over the internet.

  - Private IP: Used for devices within a local network, not accessible directly from the internet.

- How do you configure a VLAN on a switch?

  - Use the following commands:

Copy

Switch(config)# vlan 10

Switch(config-vlan)# name Sales

Switch(config-vlan)# exit

Switch(config)# interface fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

- What is the purpose of the "no ip domain-lookup" command?

  - The no ip domain-lookup command disables DNS lookup, preventing the device from trying to resolve unknown commands as hostnames.

### What is Subnetting?

- Subnetting is the process of dividing a larger IP network into smaller, more manageable subnetworks (subnets).

- Purpose:

  - Efficient use of IP addresses.

  - Reduce network congestion by limiting broadcast traffic.

  - Improve security by isolating network segments.

- Example:

  - A network with the base IP 172.16.1.0/24 can be divided into smaller subnets like 172.16.1.0/25 and 172.16.1.128/25.

### Difference Between Router and Switch

| Feature | Router | Switch |
| --- | --- | --- |
| Layer | Operates at Layer 3 (Network Layer). | Operates at Layer 2 (Data Link Layer). |
| Function | Routes data between different networks. | Forwards data within the same network. |
| Addressing | Uses IP addresses to route packets. | Uses MAC addresses to forward frames. |
| Broadcast Domain | Breaks broadcast domains (each port is a separate broadcast domain). | All ports are in the same broadcast domain (unless VLANs are used). |
| Use Case | Connects different networks (e.g., LAN to WAN). | Connects devices within the same network (e.g., PCs, printers). |
| Example | Connects your home network to the internet. | Connects devices in an office LAN. |

### Key Differences in a Nutshell

- Router: Connects different networks, uses IP addresses, and operates at Layer 3.

- Switch: Connects devices within the same network, uses MAC addresses, and operates at Layer 2.

### Final Tips for Viva

- Understand the Basics: Be clear on concepts like VLANs, subnetting, routing, and switching.

- Practice Configurations: Be familiar with basic commands for VLANs, OSPF, SSH, and PAT.

- Explain Redundancy: Be able to explain how redundancy improves network reliability.

### 1. IP Addressing

- Definition: An IP address uniquely identifies a device on an IP network.

- Format: 32-bit address divided into 4 octets (e.g., 172.16.81.100).

- Binary to Decimal: Each octet ranges from 0 to 255 (e.g., 205.112.45.60 = 11001101.01110000.00101101.00111100).

- Parts of an IP Address:

  - Network Portion: Identifies the network.

  - Host Portion: Identifies the device within the network.

### 2. Classful Addressing

- Classes: IPv4 addresses are divided into 5 classes:

  - Class A: 0.0.0.0 to 127.255.255.255 (Default mask: 255.0.0.0 or /8).

  - Class B: 128.0.0.0 to 191.255.255.255 (Default mask: 255.255.0.0 or /16).

  - Class C: 192.0.0.0 to 223.255.255.255 (Default mask: 255.255.255.0 or /24).

  - Class D: 224.0.0.0 to 239.255.255.255 (Used for multicast).

  - Class E: 240.0.0.0 to 255.255.255.255 (Reserved for experimental use).

### 3. Subnetting

- Definition: Dividing a large network into smaller subnets.

- Purpose:

  - Efficient IP address usage.

  - Reduce network congestion.

  - Improve security by isolating segments.

- Subnet Mask: 32-bit mask that separates the network and host portions.

  - Network Bits: Set to 1.

  - Host Bits: Set to 0.

  - Example: IP = 192.168.1.10, Subnet Mask = 255.255.255.0 → Network = 192.168.1, Host = 10.

### 4. CIDR (Classless Inter-Domain Routing)

- Definition: Allows flexible allocation of IP addresses by using variable-length subnet masks (VLSM).

- Slash Notation: Represents the number of network bits (e.g., /24 for Class C).

- Example:

  - Class A: /8 (255.0.0.0).

  - Class B: /16 (255.255.0.0).

  - Class C: /24 (255.255.255.0).

### 5. Subnetting Class C Addresses

- Default Mask: 255.255.255.0 (/24).

- Host Bits: 8 bits available for hosts.

- Example:

  - IP: 192.168.1.0/26 (borrowed 2 bits for subnets).

  - Subnets: 4 (2^2).

  - Hosts per subnet: 64 (2^6 - 2).

### 6. Subnetting Class B Addresses

- Default Mask: 255.255.0.0 (/16).

- Host Bits: 16 bits available for hosts.

- Example:

  - IP: 172.16.0.0/20 (borrowed 4 bits for subnets).

  - Subnets: 16 (2^4).

  - Hosts per subnet: 4094 (2^12 - 2).

### 7. Subnetting Class A Addresses

- Default Mask: 255.0.0.0 (/8).

- Host Bits: 24 bits available for hosts.

- Example:

  - IP: 10.0.0.0/12 (borrowed 4 bits for subnets).

  - Subnets: 16 (2^4).

  - Hosts per subnet: 1,048,574 (2^20 - 2).

### 8. Key Formulas

- Number of Subnets: 2x2x (x = borrowed host bits).

- Total Hosts per Subnet: 2y2y (y = remaining host bits).

- Valid Hosts per Subnet: 2y−22y−2 (subtract network and broadcast addresses).

- Block Size: 256−subnet mask256−subnet mask (e.g., 256 - 192 = 64).

### 9. Example Calculations

- IP: 192.168.1.0/26

  - Subnet Mask: 255.255.255.192.

  - Subnets: 4 (2^2).

  - Hosts per Subnet: 64 (2^6 - 2).

  - Valid Host Range: 192.168.1.1 to 192.168.1.62.

  - Broadcast Address: 192.168.1.63.

### 10. Key Terms

- Network Address: First address of a subnet (e.g., 192.168.1.0).

- Broadcast Address: Last address of a subnet (e.g., 192.168.1.63).

- Valid Hosts: Addresses between network and broadcast addresses.

### 11. Summary

- IP Addressing: Identifies devices on a network.

- Subnetting: Breaks large networks into smaller subnets.

- CIDR: Allows flexible IP allocation using VLSM.

- Classful Addressing: Divides IP space into classes (A, B, C, D, E).

- Key Formulas: Subnets, hosts, block size, and valid hosts.

### Quick Tips for Viva

- Understand Binary to Decimal Conversion: Essential for subnetting.

- Memorize Key Formulas: Subnets, hosts, and block size.

- Practice Examples: Work through subnetting problems.

- Explain Concepts Clearly: Be ready to explain subnetting and CIDR.

Here’s a concise explanation of the terms and the process of calculating subnet masks:

### Key Concepts

- Network Address:

  - The first IP address in a subnet.

  - Used to identify the subnet itself.

  - Example: For 172.16.1.0/25, the network address is 172.16.1.0.

- Host Range:

  - The range of usable IP addresses in a subnet that can be assigned to devices (hosts).

  - Excludes the Network Address and the Broadcast Address.

  - Example: For 172.16.1.0/25, the host range is 172.16.1.1 to 172.16.1.126.

- Broadcast Address:

  - The last IP address in a subnet.

  - Used to send messages to all devices within the subnet.

  - Example: For 172.16.1.0/25, the broadcast address is 172.16.1.127.

### How Subnet Masks Work

A Subnet Mask divides an IP address into two parts:

- Network Part: Identifies the network.

- Host Part: Identifies devices (hosts) within the network.

#### Example: 255.255.255.128 (/25)

- /25 means the first 25 bits are for the network, and the remaining 7 bits are for hosts.

- Total IPs in a subnet = 27=1282^7 = 128 (since 7 host bits remain).

- Usable IPs = 128−2=126128 - 2 = 126 (subtract 2 for Network and Broadcast addresses).

### Subnet Mask Calculation

The subnet mask determines how many IPs are available in a subnet. Common subnet masks:

| CIDR | Subnet Mask | Total IPs | Usable IPs | Purpose |
| --- | --- | --- | --- | --- |
| /24 | 255.255.255.0 | 256 | 254 | Standard subnet for large networks. |
| /25 | 255.255.255.128 | 128 | 126 | Divides /24 into two equal subnets. |
| /28 | 255.255.255.240 | 16 | 14 | Small subnets for specific needs. |
| /30 | 255.255.255.252 | 4 | 2 | Point-to-point links. |

### Steps to Calculate a Subnet

- Decide the number of hosts needed:

  - Each department or connection needs enough IPs for devices.

- Choose the closest subnet mask:

  - Match the number of required hosts to a subnet that provides enough usable IPs.

- Assign subnets sequentially:

  - Start from the base network and increment by the subnet size.

### Worked Example

#### Given:

- Base network: 172.16.1.0

- Required hosts for Sales & Marketing: Up to 126.

#### Step 1: Choose subnet mask

- Closest subnet: /25 (128 total IPs, 126 usable).

#### Step 2: Calculate details for 172.16.1.0/25

- Network Address: 172.16.1.0

- Host Range: 172.16.1.1 to 172.16.1.126

- Broadcast Address: 172.16.1.127

#### Step 3: Move to the next subnet for HR

- Start after broadcast address: 172.16.1.128/25.

### Short Notes on VLAN Configuration and Verification

### 1. Displaying VLANs on S1

- Command: show vlan brief

  - Displays all VLANs configured on the switch.

### 2. Creating and Naming VLANs on S1

- Steps:

  - Enter global configuration mode: S1# configure terminal

  - Create VLAN 10: S1(config)# vlan 10

  - Name VLAN 10: S1(config-vlan)# name Faculty/Staff

- Repeat for Other VLANs:

  - VLAN 20: name Students

  - VLAN 30: name Guest(Default)

  - VLAN 99: name Management&Native

  - VLAN 150: name VOICE

### 3. Creating VLANs on S2 and S3

- Steps:

  - Repeat the same VLAN creation process on S2 and S3.

### 4. Assigning VLANs to Ports on S2

- Steps:

  - Enter interface configuration mode: S2(config)# interface f0/11

  - Set port mode to access: S2(config-if)# switchport mode access

  - Assign VLAN 10: S2(config-if)# switchport access vlan 10

- Repeat for Other Ports:

  - VLAN 20: interface f0/18, switchport access vlan 20

  - VLAN 30: interface f0/6, switchport access vlan 30

### 5. Assigning VLANs to Ports on S3

- Steps:

  - Repeat the same VLAN assignment process on S3.

  - VOICE VLAN:

    - Enter interface configuration mode: S3(config)# interface f0/11

    - Enable QoS trust: S3(config-if)# mls qos trust cos

    - Assign VOICE VLAN: S3(config-if)# switchport voice vlan 150

### 6. Verifying Connectivity

- Command: ping between PC1 (172.17.10.21) and PC4 (172.17.20.25).

- Expected Result: Pings should fail because PC1 and PC4 are in different VLANs (VLAN 10 and VLAN 20), and inter-VLAN routing is not configured.

### 7. Explanation of Ping Failure

- Reason: VLANs segment the network, and devices in different VLANs cannot communicate without a Layer 3 device (router or Layer 3 switch) to route traffic between VLANs.

- Solution: Configure inter-VLAN routing to allow communication between VLANs.

### Key Commands Summary

- Display VLANs: show vlan brief

- Create VLANs:

  - vlan <ID>

  - name <VLAN_NAME>

- Assign VLANs to Ports:

  - interface <port>

  - switchport mode access

  - switchport access vlan <ID>

- Assign VOICE VLAN:

  - mls qos trust cos

  - switchport voice vlan <ID>

- Verify Connectivity: ping <IP>

### Quick Tips for Viva

- Understand VLAN Segmentation: VLANs isolate traffic, and devices in different VLANs cannot communicate without routing.

- Practice Commands: Be familiar with VLAN creation, assignment, and verification commands.

- Explain Inter-VLAN Routing: Be ready to explain why pings fail between VLANs and how to fix it.

### What is an Interface?

### 1. Definition

- An interface is a physical or logical connection point on a network device (such as a router, switch, or computer) that allows the device to communicate with other devices in a network.

- Interfaces are used to send and receive data packets between devices.

### 2. Types of Interfaces

- Physical Interfaces:

  - Ethernet Ports: Used for wired connections (e.g., FastEthernet, GigabitEthernet).

    - Example: FastEthernet 0/1, GigabitEthernet 0/0.

  - Serial Ports: Used for WAN (Wide Area Network) connections.

    - Example: Serial 0/0/0.

  - Wireless Interfaces: Used for Wi-Fi connections.

    - Example: WLAN0.

- Logical Interfaces:

  - VLAN Interfaces: Used for inter-VLAN routing.

    - Example: VLAN 10.

  - Loopback Interfaces: Virtual interfaces used for testing and management.

    - Example: Loopback 0.

  - Tunnel Interfaces: Used for VPNs (Virtual Private Networks).

    - Example: Tunnel 0.

### 3. Key Functions of an Interface

- Connectivity: Provides a connection point for devices to communicate.

- Data Transmission: Sends and receives data packets.

- Configuration: Allows network administrators to configure settings like IP addresses, VLANs, speed, and duplex.

- Security: Can be configured with access control lists (ACLs) to filter traffic.

### 4. Interface Configuration

- Access Mode: Used for connecting end devices (e.g., PCs, printers). Carries traffic for a single VLAN.

  - Example: switchport mode access.

- Trunk Mode: Used for connecting switches. Carries traffic for multiple VLANs.

  - Example: switchport mode trunk.

- IP Address Assignment: Assigns an IP address to the interface for routing purposes.

  - Example: ip address 192.168.1.1 255.255.255.0.

- VLAN Assignment: Assigns the interface to a specific VLAN.

  - Example: switchport access vlan 10.

### 5. Example: Configuring an Interface

- Enter Global Configuration Mode:

  - Switch# configure terminal

- Enter Interface Configuration Mode:

  - Switch(config)# interface FastEthernet 0/1

- Set Port Mode:

  - Switch(config-if)# switchport mode access

- Assign VLAN:

  - Switch(config-if)# switchport access vlan 10

- Enable the Interface:

  - Switch(config-if)# no shutdown

### 6. Common Interface Commands

- Show Interface Status:

  - show interface status: Displays the status of all interfaces.

- Show IP Interface Brief:

  - show ip interface brief: Displays a summary of IP addresses assigned to interfaces.

- Enable/Disable Interface:

  - no shutdown: Enables the interface.

  - shutdown: Disables the interface.

### 7. Key Points to Remember

- Physical vs. Logical: Physical interfaces are hardware-based, while logical interfaces are software-based.

- Access vs. Trunk: Access ports carry traffic for a single VLAN, while trunk ports carry traffic for multiple VLANs.

- IP Addressing: Interfaces can be assigned IP addresses for routing purposes.

- VLAN Assignment: Interfaces can be assigned to VLANs for network segmentation.

### 8. Quick Tips for Viva

- Understand Interface Types: Know the difference between physical and logical interfaces.

- Practice Configuration: Be familiar with commands like interface, switchport mode access, and ip address.

- Explain VLANs: Be ready to explain how VLANs are assigned to interfaces.

### What is a Switchport?

### 1. Definition

- A switchport is a physical interface on a network switch that connects to other devices, such as computers, printers, or other switches.

- It is used to forward data packets between devices within a local area network (LAN).

### 2. Types of Switchports

- Access Port:

  - Purpose: Connects end devices (e.g., PCs, printers) to the switch.

  - Traffic: Carries traffic for a single VLAN.

  - Configuration:

    - switchport mode access: Sets the port as an access port.

    - switchport access vlan <VLAN_ID>: Assigns the port to a specific VLAN.

  - Example:

Copy

Switch(config)# interface FastEthernet 0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

- Trunk Port:

  - Purpose: Connects switches to other switches or routers.

  - Traffic: Carries traffic for multiple VLANs.

  - Configuration:

    - switchport mode trunk: Sets the port as a trunk port.

    - switchport trunk allowed vlan <VLAN_LIST>: Specifies which VLANs are allowed on the trunk.

  - Example:

Copy

Switch(config)# interface GigabitEthernet 0/1

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20,30

- Voice Port:

  - Purpose: Connects VoIP phones and prioritizes voice traffic.

  - Traffic: Carries both data and voice traffic (separate VLANs).

  - Configuration:

    - switchport voice vlan <VLAN_ID>: Assigns a voice VLAN to the port.

    - mls qos trust cos: Enables QoS (Quality of Service) for voice traffic.

  - Example:

Copy

Switch(config)# interface FastEthernet 0/11

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# switchport voice vlan 150

Switch(config-if)# mls qos trust cos

### 3. Key Functions of a Switchport

- Connectivity: Provides a connection point for devices to communicate.

- VLAN Assignment: Assigns devices to specific VLANs for network segmentation.

- Traffic Forwarding: Forwards data packets between devices within the same VLAN or across VLANs (via trunk ports).

- Security: Can be configured with port security to restrict access to authorized devices.

### 4. Common Switchport Commands

- Show Switchport Status:

  - show interface status: Displays the status of all switchports.

  - show vlan brief: Displays VLAN assignments for all ports.

- Enable/Disable Port:

  - no shutdown: Enables the port.

  - shutdown: Disables the port.

- Port Security:

  - switchport port-security: Enables port security.

  - switchport port-security maximum <number>: Sets the maximum number of MAC addresses allowed on the port.

  - switchport port-security violation shutdown: Shuts down the port if a violation occurs.

### 5. Key Points to Remember

- Access Ports: Used for connecting end devices and carry traffic for a single VLAN.

- Trunk Ports: Used for connecting switches and carry traffic for multiple VLANs.

- Voice Ports: Used for VoIP phones and prioritize voice traffic.

- VLAN Assignment: Ensures devices are grouped logically, improving network efficiency and security.

### 6. Quick Tips for Viva

- Understand Port Types: Know the difference between access, trunk, and voice ports.

- Practice Configuration: Be familiar with commands like switchport mode access, switchport mode trunk, and switchport voice vlan.

- Explain VLANs: Be ready to explain how VLANs are assigned to switchports

### Short Notes on VLAN Trunking Protocol (VTP)

### 1. What is VTP?

- Definition: VTP is a Cisco proprietary protocol used to synchronize VLAN information across switches in the same VTP domain.

- Purpose: Simplifies VLAN management by propagating VLAN additions, deletions, and changes across the network.

### 2. VTP Requirements

- Same VTP Version: All switches must use the same VTP version.

- Same Domain Name: All switches must be part of the same VTP domain.

- Server Mode: At least one switch must be in VTP server mode.

- Matching Authentication: If a password is set, it must match on all switches.

### 3. VTP Modes

- VTP Server Mode:

  - Function: Can create, modify, and delete VLANs.

  - Propagation: Sends VLAN changes to other switches in the domain.

  - Command: vtp mode server.

- VTP Client Mode:

  - Function: Cannot create, modify, or delete VLANs.

  - Propagation: Receives and forwards VLAN updates.

  - Command: vtp mode client.

- VTP Transparent Mode:

  - Function: Does not participate in VTP but forwards VTP advertisements.

  - Propagation: Can create and delete VLANs locally, but changes are not propagated.

  - Command: vtp mode transparent.

### 4. Configuring VTP

- Set VTP Mode:

  - Server: S1(config)# vtp mode server

  - Client: S2(config)# vtp mode client

  - Transparent: S3(config)# vtp mode transparent

- Set VTP Domain Name:

  - S1(config)# vtp domain Lab4

- Set VTP Password:

  - S1(config)# vtp password cisco

### 5. Configuring Trunking and Native VLAN

- Set Trunk Mode:

  - S1(config)# interface range fa0/1-5

  - S1(config-if-range)# switchport mode trunk

- Set Native VLAN:

  - S1(config-if-range)# switchport trunk native vlan 99

- Enable Interface:

  - S1(config-if-range)# no shutdown

### 6. Configuring VLANs on VTP Server

- Create VLANs:

  - S1(config)# vlan 99

  - S1(config-vlan)# name management

  - S1(config-vlan)# exit

  - Repeat for VLANs 10, 20, and 30.

- Verify VLANs:

  - S1# show vlan brief

### 7. Verifying VTP and VLAN Distribution

- Check VTP Status:

  - S1# show vtp status

- Verify VLANs on Client Switches:

  - S2# show vlan brief

  - S3# show vlan brief

### 8. Key Points to Remember

- VTP Server: Manages VLANs and propagates changes.

- VTP Client: Receives VLAN updates but cannot make changes.

- VTP Transparent: Does not participate in VTP but forwards updates.

- Trunking: Allows multiple VLANs to traverse a single link.

- Native VLAN: VLAN used for untagged traffic on a trunk port.

### Short Notes on Switch Configuration and Verification

### 1. Enter Privileged EXEC Mode

- Command: enable

- Prompt Change: Switch> → Switch#

### 2. Examine Current Switch Configuration

- Command: show running-config

- Questions:

  - Fast Ethernet Interfaces: Check the number of FastEthernet interfaces.

  - Gigabit Ethernet Interfaces: Check the number of GigabitEthernet interfaces.

### 3. Assign a Name to the Switch

- Commands:

  - configure terminal

  - hostname S1

  - exit

- Result: Prompt changes to S1#.

### 4. Secure Access to the Console Line

- Commands:

  - configure terminal

  - line console 0

  - password letmein

  - login

  - exit

- Purpose: Protects console access with a password.

### 5. Verify Console Access Security

- Steps:

  - Exit privileged EXEC mode: exit

  - Press Enter and enter the password when prompted.

- Result: Access is secured with the password letmein.

### 6. Secure Privileged Mode Access

- Commands:

  - enable

  - configure terminal

  - enable password c1$c0

  - exit

- Purpose: Protects privileged EXEC mode with a password.

### 7. Verify Privileged Mode Access Security

- Steps:

  - Exit privileged EXEC mode: exit

  - Press Enter and enter the password when prompted.

- Result: Access is secured with the password c1$c0.

### 8. Configure an Encrypted Password for Privileged Mode

- Commands:

  - config t

  - enable secret itsasecret

  - exit

- Purpose: Replaces the plaintext enable password with an encrypted enable secret.

### 9. Configure a Message of the Day (MOTD) Banner

- Commands:

  - config t

  - banner motd "This is a secure system. Authorized Access Only!"

  - exit

- Purpose: Displays a message when someone logs into the switch.

### 10. Save Configuration to NVRAM

- Command: copy running-config startup-config

- Purpose: Saves the current configuration to NVRAM for persistence after reboot.

### 11. Examine Flash Memory

- Commands:

  - show flash

  - dir flash:

- Purpose: Displays the contents of the switch's flash memory.

### 12. Configure an IP Address on PC-A

- Steps:

  - Assign IP address: 192.168.1.10/24

  - Assign IPv6 address: 2001:db8:acad:3/64

  - Assign Link-Local address: fe80::3

### 13. Verify and Test Network Connectivity

- Commands:

  - ping <IP Address>: Tests connectivity to another device.

  - show ip interface brief: Verifies IP configuration on the switch.

- Purpose: Ensures that the network is functioning correctly.

### Key Points to Remember

- Privileged EXEC Mode: Accessible via enable command.

- Configuration Mode: Entered via configure terminal.

- Security:

  - Console access: line console 0, password, login.

  - Privileged mode: enable password, enable secret.

- MOTD Banner: banner motd.

- Saving Configuration: copy running-config startup-config.

- Verification: Use show commands to verify configurations.

### Quick Tips for Viva

- Understand Modes: Know the difference between user EXEC, privileged EXEC, and configuration modes.

- Practice Commands: Be familiar with enable, configure terminal, hostname, password, login, enable secret, banner motd, and copy running-config startup-config.

- Explain Security: Be ready to explain how to secure console and privileged mode access.

### Short Notes on Inter-VLAN Routing

### 1. Introduction to Inter-VLAN Routing

- Purpose: Enables communication between devices in different VLANs.

- Problem: By default, devices in different VLANs cannot communicate because VLANs segment broadcast domains.

- Solution: Use a router to forward traffic between VLANs.

### 2. Methods of Inter-VLAN Routing

- Traditional Inter-VLAN Routing:

  - Description: Uses multiple router interfaces, one for each VLAN.

  - Configuration:

    - Router interfaces are configured as default gateways for each VLAN.

    - Switch ports connected to the router are configured as access ports in their respective VLANs.

  - Example:

    - VLAN 20: Fa0/0 on router configured as 192.168.20.1/24.

    - VLAN 30: Fa0/1 on router configured as 192.168.30.1/24.

- Router-on-a-Stick:

  - Description: Uses a single router interface with subinterfaces for each VLAN.

  - Configuration:

    - Router interface is configured as a trunk port.

    - Subinterfaces are created for each VLAN.

  - Example:

    - VLAN 20: Subinterface Fa0/0.20 configured as 192.168.20.1/24.

    - VLAN 30: Subinterface Fa0/0.30 configured as 192.168.30.1/24.

### 3. Traditional Inter-VLAN Routing Configuration

- Switch Configuration:

  - Assign switch ports to VLANs:

    - interface Fa0/1

    - switchport mode access

    - switchport access vlan 20

    - interface Fa0/2

    - switchport mode access

    - switchport access vlan 30

- Router Configuration:

  - Configure router interfaces as default gateways:

    - interface Fa0/0

    - ip address 192.168.20.1 255.255.255.0

    - no shutdown

    - interface Fa0/1

    - ip address 192.168.30.1 255.255.255.0

    - no shutdown

### 4. Router-on-a-Stick Configuration

- Switch Configuration:

  - Configure trunk port:

    - interface Fa0/1

    - switchport mode trunk

- Router Configuration:

  - Create subinterfaces for each VLAN:

    - interface Fa0/0.20

    - encapsulation dot1Q 20

    - ip address 192.168.20.1 255.255.255.0

    - interface Fa0/0.30

    - encapsulation dot1Q 30

    - ip address 192.168.30.1 255.255.255.0

### 5. Verification and Testing

- Ping Test:

  - Use the ping command to test connectivity between devices in different VLANs.

  - Example: ping 192.168.30.10 from a device in VLAN 20.

- Routing Table:

  - Check the router’s routing table to verify routes:

    - show ip route

### 6. Key Points to Remember

- VLAN Segmentation: VLANs logically segment a network into different broadcast domains.

- Inter-VLAN Routing: Enables communication between VLANs using a router.

- Traditional vs. Router-on-a-Stick:

  - Traditional uses multiple interfaces.

  - Router-on-a-Stick uses a single interface with subinterfaces.

- Configuration:

  - Assign VLANs to switch ports.

  - Configure router interfaces or subinterfaces as default gateways.

### 7. Quick Tips for Viva

- Understand VLANs: Know how VLANs segment networks and why inter-VLAN routing is needed.

- Practice Configurations: Be familiar with configuring both traditional and router-on-a-stick methods.

- Explain the Process: Be ready to explain how traffic flows between VLANs using a router.

- Verification: Use ping and show ip route to verify configurations.