SIR SYED UNIVERSITY OF ENGINEERING & TECHNOLOGY

COMPUTER SCIENCE & INFORMATION TECHNOLOGY DEPARTMENT

Fall, 2023

Computer Organization & Assembly Language (CS-215T)

Assignment # 3

Semester:	3rd 								Batch: 2022F

Announced Date: 24-01-2024							Due Date: 26-01-2024

Total Marks:	04								Marks Obtained:

Instructor Name: Dr. Abdul Hameed Pitafi

| CLO # | Course Learning Outcomes (CLOs) | PLO Mapping | Bloom’s Taxonomy |
| --- | --- | --- | --- |
| CLO 3 | Apply the elements and design concepts of cache memory using different mapping techniques to improve the performance of computer system. | PLO_3 (Problem Analysis) | C3 (Applying) |

Q1. Consider a 8-way set associative mapping, and direct mapping with 16 cache blocks. The CPU requests the following main memory in the order- 0, 255, 1, 4, 3, 8, 133, 159, 216, 129, 63, 8, 48, 32, 73, 92, and 155. Assume that LRU replacement policy is used.

i.	Calculate the type of Miss.

ii.	Calculate the number of Hit.

iii.	Calculate Hit Ratio.

iv.	Calculate Miss Ratio.

v.	Show that which block of main memory will not be present in the cache at the end of sequence.

Q2. Consider a computer with the following characteristics: total of 1Mbyte of main memory; word size of 1 byte; block size of 16 bytes; and cache size of 64 Kbytes.

(i)	For the main memory addresses of F0010, 01234, and CABBE, compute the corresponding tag, cache line address, and word offsets for a direct-mapped cache.

(ii)	Predict any two main memory addresses with different tags that map to the same cache slot for a

direct-mapped cache.

(iii) For the main memory addresses of F0010 and CABBE, show the corresponding tag and offset values for a fully-associative cache.