Computer Organization And Assembly Language

Lab#02

Assembler And Program Execution Commands Using Debug Utility:

Mount:

Entering Debug:

Task#02:

Give answers to the following questions after unassemble the code: Why the values 0200H and 0400H are written as 0002 and 0004, in the opcode, respectively?

Solution:

0200H Operand Instruction:

In little-endian format, the least significant byte is stored at the lower memory address. For the 16-bit value 0200H, the least significant byte is 00H, and the most significant byte is 02H. In memory, this is stored as 0002 (least significant byte comes first).

0400H Operand Instruction:

Similarly, for the 16-bit value 0400H, the least significant byte is 00H, and the most significant byte is 04H. In little-endian format, it is stored as 0004.

Write the number of bytes and the value of IP register taken by each instruction:

- 0200H Operand Instruction:

  - Opcode: 0002

  - Bytes: 3 (assuming MOV instruction)

  - IP After: 0103H

- 0400H Operand Instruction:

  - Opcode: 0004

  - Bytes: 3 (assuming MOV instruction)

  - IP After: 0106H

Task#03:

Assemble a program using the DEBUG programming utility to move the decimal values in the registers, as given below:

AX = (5431)10

BX = (9321) 10

CX = (45032) 10

DX = (23102) 10

(Hint: First convert the above numbers in Hexadecimal system):

Solution:

Task#04:

- Apply the program control command T to execute the code give in Activity -1 & 3. Capture the screenshot. Write down and analyze the values of each registers including IP and Flag registers.

Solution:

AX = 1537H

BX = 2469H

CX = AFE8H

DX = 5A3EH

SP = 00FDH

BP = 0000H

SI = 0000H

DI = 0000H

DS = O73F

ES = 003FH

SS = 073F

CS = 673FH

IP = 010CH

Flag Registers: NV UP EI PL NZ NA PO NC

Task#05:

Apply the program control command G to execute the code give in Activity -1 & 3. Capture the screenshot. Write down and analyze the values of each registers including IP and Flag registers.

Solution:

AX = 0000H

BX = 0000H

CX = 0000H

DX = 6400H

SP = 00FDH

BP = 0000H

SI = 0000H

DI = 0000H

DS = O73F

ES = 073F

SS = 073F

CS = 073F

IP = 0106H

Flag Registers: NV UP EI PL NZ NA PO NC